POSEIDON v46.3

Revize Nihai Kapsam ve Ürün Karar Kaydı

NOZZLE benchmark sonrası kontrollü scope kararı

# 1. Ana Ürün Kararı

POSEIDON = Ship + Tug Safe Operations, Compliance & Readiness Platform

POSEIDON, geniş kapsamlı bir maritime ERP olarak konumlandırılmayacaktır. Ürün; gemi ve römorkörün operasyonel hazırlığı, güvenliği, mevzuat uygulanabilirliği, teknik güvenilirliği, kanıt zinciri ve insan kararını bir arada yönetmeye odaklanacaktır.

# 2. v46.3 ile Eklenen / Güçlendirilen Alanlar

| Alan | Karar | Faz | Kapsam |
| --- | --- | --- | --- |
| Noon Report | ADD | MVP | Tek girişli günlük operasyon merkezi; voyage/fuel/ROB/performans ve compliance verisini besler. |
| Deck Daily Log | ADD | MVP | Watch/navigation/port/cargo/handover kayıtları. |
| Engine Daily Log | ADD | MVP | Hours/fuel/measurements/alarms/defects/handover. |
| Fuel/Bunker/ROB/BDN | ADD | MVP | Consumption, delivery, ROB, BDN evidence ve applicable compliance fields. |
| Port Activity / SOF | ADD | Phase 2 | Vessel-side timestamped activity; Port OS değildir. |
| Operational Downtime / Loss of Hire | ADD | Phase 2 | Defect/WO/event causal trace ve claim evidence. |
| Service & Engineer Report | ADD | Phase 2 | WO/equipment service evidence ve history. |
| Pre-Survey Checklist | ADD | MVP/Phase 2 | Survey readiness, evidence ve voyage-fit. |
| Risk Assessment Workspace | ADD | MVP | Initial/residual risk, controls, barriers, approvals. |
| Risk Template Library | ADD | MVP | Fleet-standard template + revision + crew feedback. |
| Fleet High-Risk Work | ADD | Phase 2 | DPA/HSEQ exception visibility. |
| PMS Measurement/Postponement | STRENGTHEN | MVP/Phase 2 | Measurement points, threshold triggers, postponement governance. |
| Critical Spares | STRENGTHEN | MVP | Low-stock + controlled technical requisition. |
| Drydock / Survey Planner | STRENGTHEN | Phase 2 | WO/defect/certificate/survey/voyage connections. |

# 3. Ana Farklılaştırıcılar - Değiştirilmeyecek

Römorkör first-class mode: Job Intake → Assignment → Readiness → Towage Plan → Risk/Briefing → Human Release → Execution → Evidence/History.

Dynamic Applicability Engine: source/version/effective date + Flag/Class/GT/kW/build/operation/cargo/equipment/crew context.

TRUE / FALSE / UNKNOWN; kritik UNKNOWN → REVIEW_REQUIRED.

Readiness Engine: READY / CONDITIONAL / NOT_READY / REVIEW_REQUIRED + Why.

Requirement → Control → Evidence → Verification → Authorized Human Decision.

Completed ≠ Verified ≠ Effective.

Master/Tug Master onboard safety authority bypass edilemez.

Barrier Management ve Effectiveness Review.

Offline-first critical workflows; material conflictlerde silent last-write-wins yok.

AI: Detect → Explain → Recommend → Authorized Human Decision.

# 4. Bilinçli Kapsam Dışı / Entegrasyon Sınırı

| Alan | Karar | Açıklama |
| --- | --- | --- |
| Full Finance / AP-AR / Budget / FX | OUT / INTEGRATE | Harici ERP/finance sistemiyle entegre edilir. |
| Crew Payroll / Expenses | OUT / INTEGRATE | Crew eligibility POSEIDON'da; bordro/masraf HR/payroll sisteminde. |
| Full Procurement / RFQ / PO / Supplier Evaluation | OUT / INTEGRATE | Yalnız technical requisition + critical spare trigger POSEIDON'da. |
| Enterprise Warehouse / Full Inventory ERP | OUT / INTEGRATE | Yalnız teknik/critical spares stock visibility. |
| Full Chartering | OUT / INTEGRATE | Voyage/charter context okunabilir; charter-party lifecycle geliştirilmez. |
| Port/Terminal/Berth/VTS/Port Traffic Management | OUT | Yalnız gemi/römorkörün kendi operasyon bağlamı tutulur. |

# 5. Kontrollü Aday Envanteri

| Kriter | v46.3 |
| --- | --- |
| Parent baseline | v46.2 |
| Screen inventory | 196 |
| Field-level contracts | 2.316 |
| v46.3 new benchmark screens | 11 |
| Tug primary screens | 13 — retained |
| Pilot minimum | 1 real ship + 1 real tug |
| Final status | CONTROLLED CANDIDATE — not yet FINAL |

# 6. Final Olma Koşulu

Bu karar kaydı ürün scope'unu dondurur; ancak v46.3'ün FINAL baseline olması için PO/BA, Marine/Tug SME, Regulatory/Class, QA ve Engineering gate'leri ile gerçek Ship/Tug pilotlarının kapanması gerekir. Bu kapanmadan ürün sertifikalı veya tüm bağlamlarda uyumlu olarak sunulamaz.